SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA03 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100 Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | PAVITHRA K | Reg. No.: | 192411027 |
| Section / Batch: | 3RD YEAR | Date: | 10.10.2026 |

Code of Conduct I PAVITHRA K- 192411027(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student: PAVITHRA K

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Questions

1. An operator adopts a 12-cell reuse cluster. What is the co-channel reuse ratio Q = D/R for this design?

2.. A 1800 MHz carrier is used for a GSM rollout. What is its wavelength?

3.. An Abis link runs on an E1 line of 2.048 Mbps, divided into 16 kbps sub-channels. How many sub-channels does the link carry?

4.. An AMR codec produces a 244-bit speech frame every 20 ms, and each frame is sent with 40 bytes of IP/UDP/RTP headers. What bit rate does the stream use on the link?

5.. A PLMN uses a 15-digit IMSI with a 3-digit MCC and a 2-digit MNC. How many digits are left for the MSIN, and how many subscribers can the operator number?

6.. Another operator in the same country is assigned a 3-digit MNC. How many unique subscriber identities can it issue?

1.Calculate the Co-channel Reuse Ratio

Question: An operator adopts a 12-cell reuse cluster. What is the co-channel reuse ratio \(Q=D/R\) for this design?

Given Data

- Cluster size, \(N=12\)

- Co-channel reuse ratio, \(Q=D/R\)

- \(D\) = distance between co-channel cell centres

- \(R\) = radius of a cell

Formula

In a hexagonal cellular network, the co-channel reuse ratio is calculated using:

Q=DR=3N
Calculation

Substitute the cluster size \(N=12\) into the formula:

Q=3×12
Q=36
Q=6

Explanation

The co-channel reuse ratio represents the distance between two cells using the same frequency divided by the cell radius. It helps determine the separation required between co-channel cells to control interference.

A larger reuse ratio generally provides better protection against co-channel interference, but it can reduce the number of channels allocated to each cell when the total spectrum is fixed.

Result

The co-channel reuse ratio for a 12-cell cluster is 6. Therefore, the reuse distance is six times the cell radius.

2. Calculate the Wavelength of a GSM Carrier

Question: A 1800 MHz carrier is used for a GSM rollout. What is its wavelength?

Given Data

Carrier frequency, f=1800 MHz

Speed of light, c=3×108 m/s

Formula

The wavelength of an electromagnetic wave is calculated using:

λ=cf
Where:

- lambda = wavelength in metres

- c = speed of light

- f = carrier frequency in hertz

Step 1: Convert the frequency

Since 1MHz = 10^6Hz,

f=1800×106 Hz

f=1.8×109 Hz

Step 2: Substitute into the formula

λ=3×1081.8×109

λ=0.1667 m

Convert metres into centimetres:

λ=0.1667×100

λ≈16.67 cm

Explanation

Wavelength is the distance travelled by a wave during one complete cycle. It is inversely proportional to frequency, meaning that a higher carrier frequency has a shorter wavelength.

In mobile communication, wavelength affects antenna dimensions, radio propagation and the interaction of signals with buildings and other obstacles.

Result

The wavelength of the 1800 MHz GSM carrier is approximately 0.1667 metres or 16.67 centimetres.

3. Calculate the Number of Sub-channels in an Abis Link

Question: An Abis link runs on an E1 line of 2.048 Mbps, divided into 16 kbps sub-channels. How many sub-channels does the link carry?

Given Data

- Total E1 data rate = 2.048 Mbps

- Data rate per sub-channel = 16 kbps

Formula

Number of sub-channels=Total data rateData rate per sub-channel

Step 1: Convert Mbps into kbps

Since 1Mbps=1000 kbps

2.048 Mbps=2048 kbps

Step 2: Calculate the number of sub-channels

Number of sub-channels=204816

Number of sub-channels=128

Explanation

The Abis interface connects the Base Transceiver Station (BTS) to the Base Station Controller (BSC) in a GSM network. It carries traffic and signalling information required for communication between these network components.

Dividing the total data rate by the rate allocated to each sub-channel gives the theoretical number of equal-sized 16 kbps sub-channels. This calculation assumes the entire stated line capacity is divided in that way, as specified in the question.

Result

The E1 Abis link carries 128 sub-channels, each with a data rate of 16 kbps.

4. Calculate the AMR Stream Bit Rate

Question: An AMR codec produces a 244-bit speech frame every 20 ms, and each frame is sent with 40 bytes of IP/UDP/RTP headers. What bit rate does the stream use on the link?

Given Data

- Speech frame size = 244 bits

- Frame duration = 20 ms

- IP/UDP/RTP header size = 40 byte

- 1 byte=8 bits

Formula

Bit rate=Total bits per frameFrame duration

Step 1: Convert frame duration into seconds

20 ms=201000 s

=0.02 s

Step 2: Convert the header size into bits

Header bits=40×8

=320 bits

Step 3: Calculate the total bits per frame

Total bits=244+320

=564 bits

Step 4: Calculate the bit rate

Bit rate=5640.02

=28,200 bits/s

Converting into kilobits per second:

Bit rate=28,2001000

Bit rate=28.2 kbps

Explanation

Adaptive Multi-Rate (AMR) is a speech codec used in mobile communication systems. It converts voice into digital speech frames for transmission over the network.

In packet-based voice communication, each speech frame is transmitted with protocol headers. These headers carry information required for packet delivery and sequencing. Therefore, the total transmission bit rate is higher than the speech codec's payload bit rate alone.

Result

The total AMR stream bit rate, including the given IP/UDP/RTP headers, is 28.2 kbps.

5. Calculate MSIN Digits and Subscriber Capacity

Question: A PLMN uses a 15-digit IMSI with a 3-digit MCC and a 2-digit MNC. How many digits are left for the MSIN, and how many subscribers can the operator number?

Given Data

- Total IMSI length = 15 digits

- Mobile Country Code (MCC) = 3 digits

- Mobile Network Code (MNC) = 2 digits

Formula

MSIN digits=IMSI digits-MCC digits-MNC digits

The theoretical number of possible MSIN values is:

Capacity=10number of MSIN digits

Step 1: Calculate the MSIN length

MSIN digits=15-3-2

MSIN digits=10

Step 2: Calculate subscriber capacity

Each decimal digit can take one of ten values, from 0 to 9.

For ten digits:

Capacity=1010

Capacity=10,000,000,000

Explanation

The International Mobile Subscriber Identity (IMSI) identifies a subscriber within a mobile network. It consists of three main parts:

- MCC: Identifies the country.

- MNC: Identifies the mobile network within the country.

- MSIN: Identifies the subscriber within the numbering structure.

The MSIN length depends on the total permitted IMSI length and the number of digits used by the MCC and MNC. A longer MNC leaves fewer digits available for the MSIN when the total IMSI length remains fixed.

The calculated capacity is theoretical; actual subscriber allocations may be lower because of numbering rules and reserved values.

Result

The MSIN contains 10 digits, giving a theoretical capacity of 10 billion unique MSIN values.

6. Calculate Unique Subscriber Identities for a Three-digit MNC

Question: Another operator in the same country is assigned a 3-digit MNC. How many unique subscriber identities can it issue?

Given Data

- Total IMSI length = 15 digits

- MCC = 3 digits

- MNC = 3 digits

Formula

MSIN digits=IMSI digits-MCC digits-MNC digits

Capacity=10MSIN digits

Step 1: Calculate the MSIN length

MSIN digits=15-3-3
MSIN digits=9
Step 2: Calculate the subscriber capacity

Each of the nine decimal digits can have ten possible values.

Capacity=109

Capacity=1,000,000,000

Explanation

The Mobile Network Code (MNC) distinguishes a mobile network within a country. When the MNC increases from two digits to three digits, one additional digit is used for identifying the network.

Since the total IMSI length remains 15 digits, the number of digits available for the MSIN decreases from ten to nine. Consequently, the theoretical number of possible subscriber identities also decreases by a factor of ten.

This demonstrates the relationship between MNC length, MSIN length and subscriber numbering capacity in a PLMN.

Result

The MSIN contains 9 digits, providing a theoretical capacity of 1 billion unique MSIN values, before accounting for numbering restrictions.

Overall Conclusion

These six problems demonstrate important numerical concepts in mobile computing. The co-channel reuse ratio helps evaluate frequency reuse and interference, wavelength calculations describe radio-wave properties, and Abis sub-channel calculations explain link capacity. The AMR bit-rate calculation shows the effect of protocol overhead on voice transmission. Finally, IMSI, MCC, MNC and MSIN calculations explain how mobile networks identify subscribers and determine theoretical numbering capacity.